Deployment Guide

BIG-IP on KVM on Equinix Metal

April 14, 2022

Contents

Prerequisites	3

Enable BGP on project	4

Create layer 2 VLANs	4

Request Elastic IP Addresses	5

Deploy and Configure Ubuntu KVM hypervisor hosts	6

Install bare metal Ubuntu instances	6

Populate user data section	7

Enable BGP on Metal instances	8

Convert Metal network	9

Assign elastic IP addresses	11

Modify instance network configuration	12

Deploy BIG-IP VM	15

Configure BIG-IP	16

Configure BIRD BGP routing daemon on Ubuntu	17

Confirm BIRD BGP neighbors	18

Deploy Sample NGINX Back-end Web Servers	18

Deploy Metal Ubuntu instances	19

Populate user data section	19

Convert Metal network	19

Modify instance network configuration	19

Test Deployment	20

Introduction

With the release of Equinix Metal, enterprises now have a great option for deploying resources on bare metal at the network edge. Additionally, since Equinix Metal is integrated into the Equinix Fabric, it provides an excellent location for a centralized access endpoint into an organization’s multi-cloud environment.

However, as organizations move sensitive resources to the network edge, it is extremely critical that security moves to the edge as well. The F5 BIG-IP, with its host of application security services, is well positioned to meet this need with highly available, scalable, and secure access to edge compute services as well multi-cloud environments.

This document provides guidance for deploying a highly available and scalable application delivery infrastructure on top of Equinix Metal using the F5 BIG-IP. As shown in Figure 1, the BIG-IP tier provides advanced layer 4/7 traffic management and security services. In this configuration, the BIG-IP instances can operate in either an “active/ active” or “active/standby” mode depending upon application requirements and services utilized.

Figure 1: The BIG-IP tier provides advanced layer 4/7 traffic management and security services

# Prerequisites

The following guidance assumes that you have an Equinix Metal account, a basic understanding of the Equinix Metal platform and the deployment process. Additionally, it is assumed the reader is familiar with KVM hypervisor configuration as well as F5 BIG-IP application delivery terminology and configuration as well as two BIG-IP license keys.

A note about BGP in the Metal environment:

Equinix Metal enables BGP to be used to advertise out either an IP address block and ASN that you own or an elastic IP address block that you rent from Equinix. However, while BGP peering can be established between a Metal instance and the Equinix Metal routers, due to the Metal architecture it isn’t possible to establish BGP peering directly with the Metal routers and the BIG-IP running on the KVM hypervisor.

To overcome this limitation, this deployment uses the Ubuntu server instance as a BGP intermediary and establishes BGP peering between the BIG-IP and the BIRD routing engine on Ubuntu which is also used to establish BGP peering with the Metal routers.

The BIG-IP is configured to advertise the virtual server elastic IP address block to BIRD on Ubuntu and BIRD advertises this block to the Equinix Metal routers.

## Enable BGP on project

This deployment uses BGP to advertise the BIG-IP virtual server IP addresses to the world and BGP must be enabled at the project level using Local BGP option. BGP also needs to be enabled on each Metal server and you will do that in a later step.

Figure 2: BGP must be enabled at the project level using Local BGP option

## Create layer 2 VLANs

You first create four layer 2 VLANs in the Equinix Metal location where all the servers reside, as shown in Figure 3. Name the VLANs as desired and note the VLAN number that is assigned to each VLAN because it will be needed when configuring the Ubuntu network settings in a later step.

Figure 3: Create four layer 2 VLANs in the Equinix Metal location where all the servers reside.

Once all of the VLANs have been created, the list of VLANs will look similar to what is shown in Figure 4.

Figure 4: Once all of the VLANs have been created, the list of VLANs will look similar to this.

## Request Elastic IP Addresses

Equinix Metal elastic IP addresses are public IPv4 addresses that users request and rent by the hour. For this deployment, you will need a total of five elastic IP addresses: two for each BIG-IP management interface and one for the BIG-IP virtual server address that will be advertised out to the world using BGP.

More specifically, you will need to request a Public IPv4 address block that is a /31 in CIDR notation for each BIG-IP as well as a Public IPv4 address block that is a /32 in CIDR notation. These addresses will be used later in the deployment process.

The elastic IP addresses are location specific and you will need to make sure that you request them for the same location that your Metal you intend to deploy your Metal instances. It’s also a good idea to provide a description for each elastic IP block to make it easier to know which block to assign to each BIG-IP.

Figure 5: For this deployment, you will need a total of five elastic IP addresses.

Once the three elastic IP address blocks have been requested, the list of addresses will look similar to what is shown in Figure 6.

Figure 6: The three elastic IP address blocks.

# Deploy and Configure Ubuntu KVM hypervisor hosts

## Install bare metal Ubuntu instances

Install two (2) Ubuntu 20.04 LTS on-demand Metal Instances. Select a size that supports hybrid and layer 2 networking modes (servers with 2x 10gbps ports); provide names and populate the Add User Data section (see Figure 7).

In the following example, the Dallas location and c3.small.x86 instance size has been selected along with Ubuntu 20.04, but you may deploy from other Equinix Metal locations.

Figure 7: Install two (2) Ubuntu 20.04 LTS on-demand Metal Instances.

## Populate user data section

Equinix Metal supports cloud-init functionality which, among other things, allows for files to be written and packages to be installed during the initial startup phase of Ubuntu. On the Metal server deployment page, enable the Add User Data section and populate with the following:

#cloud-config

package_upgrade: true

packages:

- qemu-kvm

- virt-manager

- libvirt-daemon-system

- libvirt-clients

- virtinst

- virt-viewer

- bridge-utils

- bird

- unzip

- python3-pip

runcmd:

- systemctl start libvirtd

- virsh net-undefine default

- virsh net-destroy default

- echo "vm.nr_hugepages=1200" >> /etc/sysctl.conf

- echo "net.ipv4.ip_forward=1" >> /etc/sysctl.conf

- echo "net.bridge.bridge-nf-call-ip6tables=0" >> /etc/sysctl.conf

- echo "net.bridge.bridge-nf-call-iptables=0" >> /etc/sysctl.conf

After populating the Add User Data section, press the Deploy Now button to begin the deployment process. It can take a few minutes after the servers are available in the dashboard for all the packages to be installed and for the servers to be fully available and ready to be configured. If you find that a package defined to be installed is not, wait a few minutes and check again.

## Enable BGP on Metal instances

Once the servers are up and available, BGP for IPv4 also needs be enabled on each Ubuntu server instance.

Figure 8: BGP for IPv4 needs be enabled on each Ubuntu server instance.

## Convert Metal network

When a Metal instance is first deployed, the two NICs are connected to two different switches, bonded together into a single, logical interface and supporting layer 3 only. To support the layer-2 VLANs needed for BIG-IP deployment, the Metal network configuration needs to be converted to Hybrid Bonded which supports both layer 2 and layer 3 networking.

As part of the network conversion process, you will have to assign one of the VLANs you created earlier. You can select any VLAN and you will assign the remaining three once the network conversion is complete. (See Figure 9, next page.)

Figure  9: As part of the network conversion process, you assign one of the VLANs created earlier.

After the Metal network has been converted to hybrid bonded mode, assign the three remaining VLANs to each Ubuntu instance. (See Figure 10 on the next page.)

Figure 10: Assign the three remaining VLANs to each Ubuntu instance.

## Assign elastic IP addresses

To connect to the BIG-IP management interface from outside the Metal environment, you need to assign a Public IPv4 /31 elastic IP address block that you provisioned in a previous step to each Ubuntu instance. (See Figure 11 on the next page.)

Figure 11: Assign a Public IPv4 /31 elastic IP address block (already provisioned) to each Ubuntu instance.

## Modify instance network configuration

NOTE: You will need to log in to each Ubuntu server via SSH instance using the auto-assigned Metal instance management IP address to complete this section. This management IP address does not need to be changed and is NOT the same as the BIG-IP management IP address.

This deployment example uses KVM network bridging and the network configuration of both Ubuntu instances needs to be modified to support this mode. The layer 2 VLANs are tagged and defined as a subinterface of the bond0 interface and the naming convention is bond0.<VLAN number>. For this deployment example, the tagged interfaces are bond0.1000, bond0.1001, bond0.1002 and bond0.1003. Depending on the VLAN numbers that were auto assigned, your interface names may be different, and you will need to modify as needed.

Only the BIG-IP management interface—bond0.1000 in this case—uses a public IP address while the rest of the new interfaces use RFC1918 private IP addresses.

Edit the interfaces configuration file and append the interface configuration commands to the bottom of the file, adjusting the new interface names to match your assigned VLAN numbers. There are two sets of interface configuration commands below, one for each Ubuntu instance. Be sure to use different RFC1918 IP addresses for each Ubuntu instance, while making sure that the matching interfaces are in the same subnet.

Additionally, the BIG-IP management IP address—highlighted below in red—need to be changed to match the first address of the elastic IP address blocks that you requested in a previous step.

As an example, if the /31 elastic IP address block you requested was 147.28.141.130/31, the IP address you would define on the Ubuntu network configuration would be 147.28.141.130. In a later step, you will assign the second IP address of the block—147.28.141.131—as the BIG-IP management IP address.

Ubuntu #1

vi /etc/network/interfaces

auto br0

iface br0 inet static

address <first IP of BIG-IP mgmt address block>

netmask 255.255.255.254

bridge_ports bond0.1000

bridge_stp off

bridge-fd 0

bridge_maxwait 0

auto bond0.1001

iface bond0.1001 inet manual

pre-up sleep 5

vlan-raw-device bond0

auto br1

iface br1 inet static

address 192.168.10.10

netmask 255.255.255.0

bridge_ports bond0.1001

bridge_stp off

bridge-fd 0

bridge_maxwait 0

auto bond0.1002

iface bond0.1002 inet manual

pre-up sleep 5

vlan-raw-device bond0

auto br2

iface br2 inet static

address 192.168.20.10

netmask 255.255.255.0

bridge_ports bond0.1002

bridge_stp off

bridge-fd 0

bridge_maxwait 0

auto bond0.1003

iface bond0.1003 inet manual

pre-up sleep 5

vlan-raw-device bond0

auto br3

iface br3 inet static

address 192.168.30.10

netmask 255.255.255.0

bridge_ports bond0.1003

bridge_stp off

bridge-fd 0

bridge_maxwait 0

Ubuntu #2

vi /etc/network/interfaces

auto br0

iface br0 inet static

address <first IP of BIG-IP mgmt address block>

netmask 255.255.255.254

bridge_ports bond0.1000

bridge_stp off

bridge-fd 0

bridge_maxwait 0

auto bond0.1001

iface bond0.1001 inet manual

pre-up sleep 5

vlan-raw-device bond0

auto br1

iface br1 inet static

address 192.168.10.20

netmask 255.255.255.0

bridge_ports bond0.1001

bridge_stp off

bridge-fd 0

bridge_maxwait 0

auto bond0.1002

iface bond0.1002 inet manual

pre-up sleep 5

vlan-raw-device bond0

auto br2

iface br2 inet static

address 192.168.20.20

netmask 255.255.255.0

bridge_ports bond0.1002

bridge_stp off

bridge-fd 0

bridge_maxwait 0

auto bond0.1003

iface bond0.1003 inet manual

pre-up sleep 5

vlan-raw-device bond0

auto br3

iface br3 inet static

address 192.168.30.20

netmask 255.255.255.0

bridge_ports bond0.1003

bridge_stp off

bridge-fd 0

bridge_maxwait 0

Restart networking services to enable the new configuration.

systemctl restart networking

Next, validate communication between the Ubuntu servers by pinging from one to the corresponding VLAN IP address of the other, e.g., 192.168.10.10 -> 192.168.10.20.

# Deploy BIG-IP VM

Now that the KVM hypervisor networking is properly configured, download the latest qcow2 BIG-IP image from downloads.f5.com and perform the following steps on each Ubuntu instance:

Unzip and copy the downloaded image file to the /var/lib/libvirt/images directory

Next, create BIG-IP virtual machine using virt-install utility, adjusting the image name (highlighted in red) as appropriate.

virt-install --name big-ip --ram 16384 --vcpus=8 --os-variant=centos7.0 \

--network bridge=br0,model=virtio \

--network bridge=br1,model=virtio \

--network bridge=br2,model=virtio \

--network bridge=br3,model=virtio \

--accelerate \

--disk path=/var/lib/libvirt/images/BIGIP-16.1.2.1-0.0.10.qcow2,bus=virtio,cache=none,size=96 \

--noautoconsole --noreboot --import

Start the virtual machine and also set to autostart when Ubuntu is rebooted:

virsh start big-ip
virsh autostart big-ip

Get the console number of the BIG-IP virtual machine:

virsh list

After waiting a few minutes, connect to BIG-IP console using console ID number. For example, if the number 1 was returned from the virsh list command:

virsh console 1

Login to BIG-IP and change password for root from the default. Additionally, while the admin password is also changed at the same time as the root password, it’s marked as expired and must be changed the next time the admin user logs in. To avoid having the change the admin password later, use the following TMSH commands to change it now:

tmsh modify auth password admin
tmsh save /sys config

Configure BIG-IP management interface and set IP address to second elastic IP address of the /31 used for management and set management route to the first elastic IP address of the /31 used for BIG-IP management.

For example, if the Metal elastic IP address block is 147.28.141.130/31, configure the management IP address to be 147.28.141.131 and the management route to be 147.28.141.130.

## Configure BIG-IP

Instead of using the BIG-IP web UI to configure the BIG-IP, you will use the BIG-IP CLI and TMSH commands to configure the BIG-IP instances. Below, are two sets of commands: one for BIG-IP #1 and the other for BIG-IP #2. You will need to supply a unique license key for each BIG-IP as well as adjust references to IP addresses to match the IP addresses you are using.

Once all of the TMSH commands have been entered on both BIG-IP instances, you should have an active/standby pair of BIG-IPs up and ready to go.

NOTE: The KVM console can be a little difficult to work with and you may want to use SSH to configure the BIG-IP instances instead. Also, highlighted below in red are entries that you may have to change; however, if you have used the same RFC1918 IP addresses, then the only items you will have to change are the license key and the virtual server elastic IP address block.

BIG-IP #1

tmsh modify sys global-settings hostname bigip-1.example.com

tmsh create net vlan external interfaces add {1.1}

tmsh create net vlan internal interfaces add {1.2}

tmsh create net vlan ha interfaces add {1.3}

tmsh create net self 192.168.20.11/24 vlan internal allow-service default

tmsh create net self 192.168.10.11/24 vlan external allow-service default

tmsh create net self 192.168.30.11/24 vlan ha allow-service default

tmsh modify sys global-settings gui-setup disabled

tmsh mv cm device bigip1 bigip-1.example.com

tmsh modify cm device bigip-1.example.com configsync-ip 192.168.30.11

tmsh modify cm device bigip-1.example.com unicast-address {{ ip 192.168.30.11 }}

tmsh modify sys db tmrouted.tmos.routing value enable

tmsh create net routing bgp my_bgp_config local-as 65000 neighbor add { 192.168.10.10 { remote-as 65000 } } network add { <virtual server elastic IP address block/CIDR> } graceful-restart { restart-time 120 }

tmsh modify /sys dns name-servers add { 8.8.8.8 }

tmsh modify /sys ntp servers add { pool.ntp.org }

tmsh install /sys license registration-key <license key>

tmsh save sys config

BIG-IP #2

tmsh modify sys global-settings hostname bigip-2.example.com

tmsh create net vlan external interfaces add {1.1}

tmsh create net vlan internal interfaces add {1.2}

tmsh create net vlan ha interfaces add {1.3}

tmsh create net self 192.168.20.21/24 vlan internal allow-service default

tmsh create net self 192.168.10.21/24 vlan external allow-service default

tmsh create net self 192.168.30.21/24 vlan ha allow-service default

tmsh modify sys global-settings gui-setup disabled

tmsh mv cm device bigip1 bigip-2.example.com

tmsh modify cm device bigip-2.example.com configsync-ip 192.168.30.21

tmsh modify cm device bigip-2.example.com unicast-address {{ ip 192.168.30.21 }}

tmsh modify /cm trust-domain /Common/Root add-device { device-ip 192.168.30.11 device-name bigip-1.example.com username admin password <admin password of BIG-IP #1> ca-device true }

tmsh create cm device-group devicegroup-1 devices add {bigip-1.example.com bigip-2.example.com} type sync-failover auto-sync enabled

tmsh run cm config-sync to-group devicegroup-1

tmsh modify sys db tmrouted.tmos.routing value enable

tmsh create net routing bgp my_bgp_config local-as 65000 neighbor add { 192.168.10.20 { remote-as 65000 } } network add { <virtual server elastic IP address block/CIDR> } graceful-restart { restart-time 120 }

tmsh modify /sys dns name-servers add { 8.8.8.8 }

tmsh modify /sys ntp servers add { pool.ntp.org }

tmsh install /sys license registration-key <license key>

tmsh create ltm  pool nginx members add { 192.168.20.100:http 192.168.20.110:http } monitor http

tmsh create ltm virtual nginx { destination  <virtual server elastic IP address block>:80 pool nginx ip-protocol tcp source-address-translation { type automap } translate-address enabled translate-port enabled }

tmsh save sys config

## Configure BIRD BGP routing daemon on Ubuntu

The BIRD routing daemon provides BGP routing capability and will be used to establish BGP neighbors with both the Equinix Metal routers as well as the BIG-IP instances. Equinix Metal provides a convenience script that performs the initial configuration of the BIRD routing engine. To use the script, perform the following:

git clone https://github.com/packethost/network-helpers.git

cd network-helpers

pip3 install jmespath

pip3 install -e . 
./configure.py -r bird | tee /etc/bird/bird.conf

The script configures BIRD to establish BGP neighbors with the two Equinix Metal router instances. However, BIRD needs to be configured to also establish a BGP neighbor with the BIG-IP as well. The neighbor IP address for the BIG-IP is the external VLAN self-ip address.

Modify the BIRD configuration file and add a static route to the BIG-IP external VLAN self-ip address and add the BIG-IP as a BGP neighbor

Ubuntu #1

nano vi /etc/bird/bird.conf

Locate the protocol static section and add the following between the curly braces:

route 192.168.10.11/32 via 192.168.10.10;

At the bottom of the file, add the following:

protocol bgp neighbor_v4_3 {

export filter packet_bgp;

local as 65000;

neighbor 192.168.10.11 as 65000;

}

Ubuntu #2

nano vi /etc/bird/bird.conf

Locate the protocol static section and add the following between the curly braces:

route 192.168.10.21/32 via 192.168.10.20;

At the bottom of the file, add the following:

protocol bgp neighbor_v4_3 {

export filter packet_bgp;

local as 65000;

neighbor 192.168.10.21 as 65000;

}

Save that file and restart the BIRD service:

systemctl restart bird

## Confirm BIRD BGP neighbors

Using the BIRD utility, confirm that that the two Metal routers and the BIG-IP are neighbors and that the virtual server IP address block is being advertised:

birdc show route

The output should look similar to the below (elastic IP address block highlighted for clarity):

BIRD 1.6.8 ready.

192.168.10.11/32   via 192.168.10.10 on br1 [static1 2022-02-02] ! (200)

39.178.82.246/31   via 192.168.10.10 on br1 [neighbor_v4_3 2022-02-02 from 192.168.10.11] ! (100/?) [i]

169.254.255.2/32   via 139.178.83.46 on bond0 [static1 2022-02-02] * (200)

169.254.255.1/32   via 139.178.83.46 on bond0 [static1 2022-02-02] * (200)

You may further validate that BGP neighbors have been established:

birdc show protocols

The output should look similar to the below (BIG-IP neighbor highlighted in red):

BIRD 1.6.8 ready.

name     proto    table    state  since       info

direct1  Direct   master   up     22:28:57

kernel1  Kernel   master   up     22:28:57

static1  Static   master   up     22:28:57

device1  Device   master   up     22:28:57

neighbor_v4_1 BGP      master   up     22:29:58    Established

neighbor_v4_2 BGP      master   up     22:31:01    Established

neighbor_v4_3 BGP      master   up     22:29:47    Established

# Deploy Sample NGINX Back-end Web Servers

In this section, you will deploy two new Metal instances that will host the Nginx web servers. The process to deploy and configure these two new Ubuntu instances is similar to the two servers you deployed in previous steps.

## Deploy Metal Ubuntu instances

Deploy two (2) additional Ubuntu 20.04 LTS on-demand Metal Instances. Select a size that supports hybrid and layer 2 networking modes (servers with 2x 10gbps ports); provide names and populate the Add User Data section (see below).

## Populate user data section

Equinix Metal supports cloud-init functionality which, among other things, allows for files to be written and packages to be installed during the initial startup phase of Ubuntu. On the Metal server deployment page, enable the Add User Data section and populate with the following:

#cloud-config

package_upgrade: true

packages:

- nginx

After populating the Add User Data section, press the Deploy Now button to begin the deployment process. It can take a few minutes after the servers are available in the dashboard for all the packages to be installed and for the servers to be fully available and ready to be configured. If you find that a package defined to be installed is not, wait a few minutes and check again.

## Convert Metal network

When a Metal instance is first deployed, the two NICs are connected to two different switches, bonded together in to a single, logical interface and support layer 3 only. To support the layer 2 VLANs needed for BIG-IP deployment, the Metal network configuration needs to be converted to hybrid bonded which supports both layer 2 and layer 3 networking. 

As a part of the network conversion process, you will have to assign one of the VLANs you created earlier and, for these servers, select the VLAN you defined earlier for the internal network.

## Modify instance network configuration

Edit the interfaces configuration file and append the interface configuration commands to the bottom of the file, adjusting the new interface name to match your assigned VLAN number. Be sure to use different RFC1918 IP addresses for each Ubuntu instance, while making sure that the matching interfaces are in the same subnet.

Ubuntu #1

vi /etc/network/interfaces

auto bond0.1002

iface bond0.1002 inet static

address 192.168.20.100

netmask 255.255.255.0

pre-up sleep 5

vlan-raw-device bond0

Ubuntu #2

vi /etc/network/interfaces

auto bond0.1002

iface bond0.1002 inet static

address 192.168.20.110

netmask 255.255.255.0

pre-up sleep 5

vlan-raw-device bond0

Restart networking services to enable the new configuration.

systemctl restart networking

Next, validate communication between the Ubuntu servers and the BIG-IP instances by pinging the BIG-IP internal VLAN IP addresses: 192.168.20.11 & 192.168.20.21.

# Test Deployment

At this point, you can test the entire deployment by pointing a web browser to the virtual server IP address that was defined earlier when you allocated an elastic IP address block for it:

http://<elastic IP address block>/

Congratulations! You now have a cluster that routes traffic evenly between web servers and is capable of failing over to a standby system for high availability.